STANDARD MICROSYSTEMS CORPORATION

EZStart

For Ethernet LAN Adapters

USER GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

This user guide describes:

o  Major features of EZStart

o  Program-installation procedures

o  How to use EZStart with various displays

o  How to detect and resolve I/O conflicts in non-PnP systems

o  Multiple SMC Ethernet adapters

o  Command line options

o  What macros are and how to use them

Note: EZStart supports all 8216 (Ultra), 80xx (Elite),

80xx (PLUS), and 8416 (EtherEZ) Series ISA, EISA, and Micro

Channel Ethernet adapters.

MAJOR FEATURES

Custom Test

This function includes the Basic Adapter Test and Two Node Test.

The Basic Adapter Test verifies the operation of the adapter's

electronic components. The Two Node Test verifies the adapter-to-

network connection. To run the Two Node Test, set up one PC as

an Initiator and set up another PC as a Responder.

Driver Installation

This function provides access to the .DOC files and driver files on

your SuperDisk. You need only select the Network Operating System

you are using. EZStart will allow you to copy the appropriate

drivers and information files to the Destination Directory of your

choice. EZStart also lets you view or print documentation files that

provide more information on installing the device driver.

PC Agent/SNMP

This function lets you install PC Agent/SNMP and view information

that the PC Agent/SNMP maintains. It also lets you create and modify

the PC Agent configuration file.

PROGRAM INSTALLATION

The EZStart program is comprised of four files:

o  EZSTART.EXE

o  EZSTART.HLP

o  EZSTART.ICO

o  EZSTART.PIF

Running EZStart from a Hard Disk

Run EZStart from your PC's hard disk for fastest operation:

1. Copy all the EZStart files from the EZStart disk to a directory

on your PC's hard disk. All files are required to run EZStart.

2. Change to the directory where you copied the EZStart files.

3. Type: EZStart

4. Press the Enter key.

Note: EZStart requires that the FILES=parameter in your CONFIG.SYS

file be set to 10 or greater.

Running EZStart from a Floppy Disk

If you prefer, you can use the following procedure to run EZStart

from a floppy disk:

1. Insert the SuperDisk into a disk drive.

2. Change to the floppy drive where the SuperDisk is installed.

For example, if the disk is in drive A, type A: and press Enter.

3. From the DOS prompt, type EZStart and press Enter.

Running EZStart from Microsoft Windows Desktop

For added convenience, Microsoft Windows users can use the

following procedure to add an EZStart icon to their Windows

desktop. This will allow you to double-click on the EZStart icon

to access EZStart directly from Microsoft Windows.

1. Copy all EZStart files to a Destination Directory on your

computer's hard disk.

2. From the Microsoft Windows Program Manager, select New in the

File menu.

3. Select Program Item.

4. Select the OK button

5. For the Command Line entry, type: C:\EZSTART\EZSTART.PIF

Note: If you copied EZStart to a directory other than C:\EZSTART,

specify the name of the directory where you copied EZStart.

You can use the Browse button to search for the directory

and filename.

6. Select Change Icon.

7. Select OK.

8. Select Browse and select the directory where you installed

EZStart.

9. Select the EZSTART.ICO file to install the icon on your Windows

desktop.

10. Select OK or Yes with each window, until you return to your

Windows desktop.

11. To test EZStart from the Windows desktop, double-click on the

EZStart icon to activate the EZStart program. To return to

Microsoft Windows, exit EZStart as you normally would.

For additional help on installing a program in Windows refer to

the Windows help on "Installing and Deleting Program Items."

Note: If you use EZStart with Windows for Workgroups, do not run

EZStart from the MS-DOS Prompt in the Main Group. Instead,

use the procedure described above to run EZStart from the

Windows Desktop, or exit from Windows and run EZStart from

the MS-DOS prompt.

EZSTART MOUSE SUPPORT

For mouse support, load your mouse driver via your system's

AUTOEXEC.BAT file or at the DOS prompt before running EZStart.

DISPLAYS

For monochrome displays, use the /T command line parameter to operate

in text mode. Refer to the COMMAND LINE OPTIONS section of this

document.

For VGA and SVGA displays, EZStart will appear in graphics mode by

default. If you want to run EZStart in text mode:

1. At the DOS prompt, type: EZStart /t

2. Press Enter.

I/O CONFLICTS

All SMC LAN adapters use a 32-byte I/O address space, which must be

free of conflicts with other devices. If you are using an SMC

Ethernet adapter that has a conflict with other devices in the

second half of the 32-byte I/O space, EZStart can still operate

successfully.

However, if the conflict is in the first half of the 32-byte I/O

space, EZStart will either display an error message about the I/O

conflict or display the message "No Software Configurable Ethernet

Adapter Found".

Resolve this type of conflict by removing all other adapters

except the video adapter, floppy disk controller, and your SMC

adapter. Then use Manual Setup to change the I/O base address

(refer to the SMC user guide for more information). Insert the

other adapters and run Manual Test again.

If the conflict still occurs, remove all other adapters except the

video adapter, floppy disk controller adapter, and your SMC

adapter. Then insert each adapter one at a time and run Manual

Test. When you find the adapter with the conflicting address,

change it and run Manual Test to verify that the conflict no

longer exists.

MULTIPLE SMC ETHERNET ADAPTERS

If you have more than one SMC adapter in your system, EZStart will

display all the adapters. Select the one that you want to configure

or test.

For installing multiple SMC adapters, install each adapter one at a

time and run EZStart's Automatic Setup and Test function. Turn off

your PC before installing the next adapter.

MACROS

A Macro is a recorded sequence of mouse clicks and/or keystrokes.

If you are performing repetitive tasks with many adapters, you

can create macros to automate this manual task.

For example, if you are installing the same configuration and the

same drivers on multiple adapters, you can create a macro that will

perform these configuration and installation tasks for you

automatically.

Macros are stored in files. The default name of the macro file is

EZSTART.MCR. If you desire, you can specify another macro filename

at the DOS command line. Note, however, that the file must have a

.MCR extension.

Creating a Macro

To create a macro:

1. Load EZStart.

2. Hold down the Ctrl key and press F1 to start recording your macro.

3. Perform the mouse clicks and/or keystrokes you want recorded in

this macro. The message "RECORD" appears on every screen until

you stop recording.

4. After you record your macro, hold down the Ctrl key and press F1

again to stop recording the macro. The macro you just created is

automatically saved in a macro file with the default name

EZSTART.MCR, residing in the directory where EZStart is located.

5. Change the default filename if desired. For example, typing the

following line at the DOS prompt:

RENAME EZSTART.MCR  YOURFILE.MCR

and pressing Enter changes the filename from EZSTART.MCR to

YOURFILE.MCR. Of course, if you rename the macro file, you can

rename it to YOURFILE.MCR or to any other filename you choose, so

long as the filename ends in .MCR.

Inserting Pauses

For even greater versatility, you can insert pauses into your macro.

A pause tells the macro to temporarily stop until a certain action

is performed.

For example, suppose you create a macro that installs drivers. You

can insert a pause at the point in the macro where the user is

required to specify a Destination Directory. After the user

specifies the Destination Directory, the macro can continue.

To insert a pause, hold down the Ctrl key and press F3 when

recording the macro.

Playing Back a Macro

After you create a macro, use the following steps to play it back:

1. Type: EZSTART [filename].MCR

In the above line, filename.MCR is the name of the macro file you

want to play back.

2. Press Enter.

Delayed Macro Playback

If you want to delay macro playback until sometime after you start

EZStart:

1. Type: EZSTART [filename].MCR /D

2. Press Enter.

Playback of the macro filename.MCR will be halted until you hold

down the Ctrl key and press F2.

EZStart Command Line Options

EZStart command line options are typically used in conjunction with

EZStart macros. From the DOS command line, you can use the following

command line format: to initiate EZStart, to specify a macro, and to

specify other command line options:

EZSTART [filename] [/D] [/T] [/?]

If a macro filename is specified, the macro will be played back.

The other command line options shown in the line above are described

below:

Option               Description

==================================================================

[filename]           Specifies the macro file. Filename extensions

must be ".MCR".

If 'filename.MCR' exists, the macro stored in

the file will be played back.

If 'filename.MCR' does not exist, a macro file

will be created there, containing all your

selections while EZStart is running. Ctrl-F1

turns the macro-recording process on and off.

/D                   Delays playback of the macro stored in

'filename.MCR' until you press Ctrl-F2.

/T                   Specifies that EZStart will operate in text

mode instead of graphics mode.

/?                   Display this help screen.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, EZStart, and SuperDisk are trademarks of

Standard Microsystems Corporation.  Other product and company names

are registered trademarks or trademarks of their respective holders.