STANDARD MICROSYSTEMS CORPORATION

Novell NetWare Client for Microsoft Windows NT

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the Novell NetWare Client for

Microsoft Windows NT files from the SuperDisk to your PC.

o  How to install the LAN driver in Microsoft Windows NT that is

running the Novell Netware Services for Windows NT.

Note: When using the NetWare 4.0x server driver with Windows NT,

EtherEZ adapters (8416 board types) must be operated in

Memory Mapped mode.

Note: This driver supports all 8216 (Ultra), 80xx (Elite),

80xx (PLUS), and 8416 (EtherEZ) Series ISA, EISA, and Micro

Channel Ethernet adapters.

This driver also supports the Novell 32-bit client specification,

which allows the driver to work with Windows NT, UnixWare, and

other 32-bit Intel-based operating systems. Contact Novell for

more information.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files' to

copy all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

The EZStart default for copying files is a:\, as the Destination

Directory.  Please make a backup copy of your SuperDisk before EZStart

copies files to the SuperDisk.

INSTALLING THE SMC8000 DRIVER IN Windows NT RUNNING Netware SERVICES

Important: If you are not using the SMC SuperDisk, make sure the

DISK.ID file is in the root directory of the disk you are

using, and that the OEMSETUP.INF and SMC8000.LAN files

reside in the root directory or a subdirectory on the disk

you are using.

1. Log on to your NT system as an Administrator or equivalent.

2. From the Program Manager's "Main" window, double-click on the

"Control Panel" icon.

3. Double-click on "Network". The Network Settings dialog box

appears.

4. Select "Add Software".

5. Scroll down the list box to "<Other> Requires disk from

manufacturer". Select this option followed by "Continue". You

are prompted for a disk in Drive A.

6. Insert the NetWare Client for MS Windows NT diskette into your

computer's floppy drive and select "OK".

7. Select "NetWare Services for Windows NT" followed by "OK".

8. From the Add Network Software dialog box, select "Continue".

9. From the MS Windows NT Setup dialog box, select "OK". Setup copies

NetWare files to their appropriate directories and adds them to the

installed software list.

10. From the Network Settings dialog box, select "Add Adapter".

11. Scroll down the list box to "<Other> Requires disk from

manufacturer". Select this option followed by "Continue". You are

prompted for a disk in Drive A.

12. Insert the SMC SuperDisk.

13. Enter the path where you want the SMC driver and information files

reside. For example, if these files reside on the SuperDisk in

drive A, type: A:\NW386_40.

14. Select "OK".

15. Select "ODI SMC8000 Adapter for NetWare" followed by "OK". A

dialog box appears, so you can specify the settings for your SMC

Ethernet adapter.

16. Specify the options for your SMC Ethernet adapter.

17. Select "OK". The driver you selected appears in the "Installed

Adapters List".

18. If you need both NetWare and MS Windows NT networking services,

and you are using a single SMC Ethernet adapter, proceed to step

19.

OR

If you have a separate network adapter for each networking

service, proceed to step 22.

19. From the "Installed Adapter Cards" list, select the NDIS (non-ODI)

driver for the SMC Ethernet adapter you just configured, followed

by "Remove".  This step is not necessary if you do not have the

NDIS driver installed.

20. From the Network Settings dialog box, select "Add Adapter".

21. From the list box, select "ODI Support for NDIS Protocols Adapter

(ODINSUP)" followed by "Continue". "ODI Support for NDIS

Protocols Adapter (ODINSUP)" appears in the "Installed Adapter

Cards" list. Any pre-installed MS Windows NT networking software

will now bind to this adapter.

22. From the Network Settings dialog box, select "Bindings" to view

the NetWare drive bindings.

23. After exiting the Network Settings dialog box, you are prompted

to restart your system. You must restart your system for NetWare

Services to become active.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, EZStart, and SuperDisk are trademarks of

Standard Microsystems Corporation.  Other product and company names

are registered trademarks or trademarks of their respective holders.